# Development Assessment Report (DRAFT)

DRAFT — prepared by Den Assessment Copilot. This document is a draft for a planning officer to review, edit and own. It is not a determination. Every factual claim is cited to its source; items marked [NEEDS PLANNER INPUT] require professional judgement.

## 1. Application details

| Application No | eDA0371/25 |
| --- | --- |
| Address | 26-30 McIntyre Street, Gordon NSW 2072 |
| Proposal | Alterations, additions and associated works to an approved residential flat building (amending DA to DA0038/23) under SEPP (Housing) 2021 |
| Council | Ku-ring-gai |
| Lodged | 2025-08-04 |
| Submissions | 3 |
| Recommendation | Refusal, for the reasons set out in this report. |

## 2. Executive summary

The application seeks consent for alterations, additions and associated works to an approved residential flat building (amending da to da0038/23) under sepp (housing) 2021. Assessment against the

applicable planning controls identifies 4

non-compliances with development standards (see Section 6). The draft recommendation is

refusal. [NEEDS PLANNER INPUT: confirm executive summary emphasis]

## 3. Site and locality

The site has an area of 3360.1 m2 (data/seeds/mcintyre/assessment_report.pdf —

survey). [NEEDS PLANNER INPUT: site description, surrounding development, constraints]

## 4. The proposal

Residential flat building (in-fill affordable housing, sepp (housing) 2021): 8 storeys, 53 dwellings,

9114.1 m2 GFA, maximum height 30.48 m.

Sources: assessment_report.pdf p1 (site area 3,360.1m2 by survey); assessment_report.pdf p12 (53 dwellings; 8 storeys); assessment_report.pdf p17 (landscaped area 1,572m2); assessment_report.pdf p18 (GFA 9,114.1m2, FSR 2.71:1; affordable GFA 872.9m2 = 9.5% of total); assessment_report.pdf p19 (cl 4.6 states max height 30.48m; permitted 26.4m incl 20% bonus).

## 5. Background

[NEEDS PLANNER INPUT: application history, pre-lodgement, RFIs]

## 6. Section 4.15(1)(a) — environmental planning instruments

Effective standards. SEPP (Housing) 2021 Ch 2/6 overrides the KLEP mapped controls for in-fill affordable housing here. Base FSR 2.2:1 and base height 22m apply; s16 permits up to +30% GFA/height only if >=15% of total GFA is affordable, and any bonus requires >=10% affordable (s16(2)). Council applied 22m + 20% = 26.4m as the permitted height.

### Development standards compliance table

| Control | Requirement | Proposed | Complies | Breach | Source |
| --- | --- | --- | --- | --- | --- |
| Permissibility — Residential flat buildings in Zone R4 | Permitted with consent under the Land Use Table | Residential flat buildings | YES | — | https://legislation.nsw.gov.au/view/whole/html/inforce/current/epi-2015-0134 |
| Floor Space Ratio | 2.2:1 (base; uplift only with qualifying affordable housing) | 2.71:1 (9114.1 m2 GFA / 3360.1 m2 site) | NO | +23.29% above base FSR | data/seeds/mcintyre/assessment_report.pdf p18 |
| Affordable housing component (SEPP Housing s16(2)) | >= 10.0% of total GFA to access any FSR/height bonus | 9.6% (872.9 m2 of 9114.1 m2) | NO | 0.4 percentage points below minimum | data/seeds/mcintyre/assessment_report.pdf p18 |
| Height of Building | 26.4 m (base 22.0 m incl 20% bonus) | 30.48 m (8 storeys) | NO | +4.08 m (15.5%) | data/seeds/mcintyre/assessment_report.pdf p19 |
| Site area (SEPP Housing s19, non-discretionary) | >= 450.0 m2 | 3360.1 m2 | YES | — | data/seeds/mcintyre/assessment_report.pdf p17 |
| Minimum landscaped area (SEPP Housing s19, non-discretionary) | >= 1008.0 m2 (lesser of 35m2/dwelling or 30% of site area) | 1572.0 m2 | YES | — | data/seeds/mcintyre/assessment_report.pdf p17 |
| DCP 7C.6 — maximum building length | Continuous length of a single building on any elevation <= 36 m | 52.3 m (assessment_report.pdf p57 (Building A length 52.3m)) | NO | +16.3 m | data/dcp/kdcp-part7-rfb.pdf 7C.6 p32 |
| DCP 7A.5 — site coverage | Maximum 30% of site area (subject to deep soil compliance) | Council disputes the applicant's site coverage and deep soil calculations (assessment_report.pdf p11); increase of +134.5 m2 over approved (p12). | REVIEW | — | data/dcp/kdcp-part7-rfb.pdf 7A.5 p15 |
| DCP 7A.3 — Building Setbacks | side boundary street boundary driveway street min 6.0m side setback min 6.0m side setback | [NEEDS PLANNER INPUT: measure from plans] | REVIEW | — | data/dcp/kdcp-part7-rfb.pdf 7A.3 p10 |
| DCP 7A.6 — Deep Soil Landscaping | Lots with the following sizes are to support a minimum number of tall trees capable of attaining a mature height of at least 18m on shale, transitional soils and 15m on sandstone derived soils. | [NEEDS PLANNER INPUT: measure from plans] | REVIEW | — | data/dcp/kdcp-part7-rfb.pdf 7A.6 p16 |
| DCP 7C.2 — Communal Open Spa | At least 10% of the site area is to be provided as communal open space. Each parcel of communal open space is to have a minimum dimension of 5m. | [NEEDS PLANNER INPUT: measure from plans] | REVIEW | — | data/dcp/kdcp-part7-rfb.pdf 7C.2 p25 |

### DCP compliance

[NEEDS PLANNER INPUT: DCP controls table — arrives with the DCP retriever (M3)]

## 7. Section 4.15(1)(b) — likely impacts

- Traffic generation and road network impacts — Adequate on-site parking; GTIA 2024 rates give ~8 AM / 6 PM peak trips for 53 dwellings — minor and acceptable. (assessment_report.pdf p12)

- Street parking, footpath access and blockages during construction — Manageable via Construction Traffic Management Plan condition (as per DA0038/23 condition 6) if approved. (assessment_report.pdf pp12-13)

- Acoustic impacts (operation and construction) — Acoustic report relates to a previous set of plans; forms part of refusal reason 14. Construction noise conditionable. (assessment_report.pdf p13)

- Dust impacts during construction — Conditionable, consistent with DA0038/23. (assessment_report.pdf p13)

[NEEDS PLANNER INPUT: solar access, privacy, visual bulk, stormwater, ecology]

## 8. Section 4.15(1)(c) — suitability of the site

[NEEDS PLANNER INPUT: site suitability conclusion]

## 9. Section 4.15(1)(d) — submissions

3 submissions were received. Issues raised and draft responses:

| Issue | Count | Head of consideration | Draft response | Source |
| --- | --- | --- | --- | --- |
| Traffic generation and road network impacts | 1 | 4.15(1)(b) likely impacts — traffic | Adequate on-site parking; GTIA 2024 rates give ~8 AM / 6 PM peak trips for 53 dwellings — minor and acceptable. | assessment_report.pdf p12 |
| Street parking, footpath access and blockages during construction | 2 | 4.15(1)(b) likely impacts — construction amenity | Manageable via Construction Traffic Management Plan condition (as per DA0038/23 condition 6) if approved. | assessment_report.pdf pp12-13 |
| Acoustic impacts (operation and construction) | 1 | 4.15(1)(b) likely impacts — noise | Acoustic report relates to a previous set of plans; forms part of refusal reason 14. Construction noise conditionable. | assessment_report.pdf p13 |
| Dust impacts during construction | 1 | 4.15(1)(b) likely impacts — air/amenity | Conditionable, consistent with DA0038/23. | assessment_report.pdf p13 |
| Increased density / unclear FSR and built form | 1 | 4.15(1)(a) EPIs — FSR standard | FSR calculation unclear and non-compliant; underpins refusal reason 6 (affordable housing) and reason 1 (GFA). | assessment_report.pdf p13 |

## 10. Section 4.15(1)(e) — public interest

The public interest is served by the consistent application of the relevant environmental

planning instruments. [NEEDS PLANNER INPUT: confirm]

## 11. Reasons for refusal (draft)

1. Floor Space Ratio does not comply: proposed 2.71:1 (9114.1 m2 GFA / 3360.1 m2 site) against a requirement of 2.2:1 (base; uplift only with qualifying affordable housing) (+23.29% above base FSR). Source: data/seeds/mcintyre/assessment_report.pdf p18.

2. Affordable housing component (SEPP Housing s16(2)) does not comply: proposed 9.6% (872.9 m2 of 9114.1 m2) against a requirement of >= 10.0% of total GFA to access any FSR/height bonus (0.4 percentage points below minimum). Source: data/seeds/mcintyre/assessment_report.pdf p18.

3. Height of Building does not comply: proposed 30.48 m (8 storeys) against a requirement of 26.4 m (base 22.0 m incl 20% bonus) (+4.08 m (15.5%)). Source: data/seeds/mcintyre/assessment_report.pdf p19.

4. DCP 7C.6 — maximum building length does not comply: proposed 52.3 m (assessment_report.pdf p57 (Building A length 52.3m)) against a requirement of Continuous length of a single building on any elevation <= 36 m (+16.3 m). Source: data/dcp/kdcp-part7-rfb.pdf 7C.6 p32.

## 12. Conclusion

Having regard to section 4.15 of the Environmental Planning and Assessment Act 1979, the

draft assessment supports refusal. A planning officer must review every section before

this assessment is relied upon.